Researcher-Specific Requirements

Table 17 Researcher specific requirements for animal tracking devices and the expected thresholds a product should meet, as collated from initial requirement gathering interviews and literature reviews. These are grouped into the overarching concerns: cost; attachment; variables collected; interface – design and delivery; interface – data visualisation and analysis; battery.

| Overarching Concern | # | Specific Requirement | Poor | Adequate | Excellent |
| --- | --- | --- | --- | --- | --- |
| Cost | 1 | Cost of overall system to be reduced (w/o Tag costs) | >$2000 (AUD) | $1000<Cost<$2000 | <$1000 |
|  | 2 | Cost per tag reduced | >$1000 (AUD) | $250<Cost<$1000 | <$250 |
|  | 3 | Cost of customisability and design reduced | No customisability available | Design included in pricing | Device design allows for DIY customisation (no additional cost) |
| Attachment | 4 | Available from Manufacturer | No Attachment style available from manufacturer – only tag available | 1 Attachment style available | 1+ attachment style available |
|  | 5 | Attachment Style for Peramelemorphia (and other marsupials) available | No Suitable Attachment Available |  | Suitable Attachment Available |
|  | 6 | Attachment is modifiable to suit species specific requirements | Attachment specifications are rigid and unchangeable | Attachment allows for specification changes at manufacture level | Attachment allows for specification changes in-field |
|  | 7 | Attachment reliably detaches within study period | No detachment mechanism | Natural Detachment | Programable detachment |
|  | 8 | Attachment is intuitive with clear instructions | Not intuitive, need expert guidance | Intuitive, though no clear instructions provided by manufacturer OR Unintuitive, but instructions are clearly provided and useable | Intuitive, and manufacturer provides clear instructions for attachment and use |
|  | 9 | Attachment in-field is not laborious | Requires 2 people Takes more than 15 mins post-capture | Requires 1-2 people Takes 10-15 mins post-capture | Requires 1 person Takes less than 10 mins post-capture |
|  | 10 | Attachment is weather resistant | Water and dust can easily permeate the design Fabrics used in the design absorb water | 1 of the 2 requirements listed is met | Water and dust cannot permeate design Fabrics used in the design are non-absorbent |
| Variables Collected | 11 | Positioning as standard |  |  |  |
|  | 12 | Positioning is accurate | Accuracy >20m | Accuracy 5-20m | Accuracy <5m |
|  | 13 | Accelerometer/Activity Metrics Available | Unavailable |  | Available |
|  | 14 | Proximity Metrics Available | Unavailable |  | Available |
|  | 15 | Core Temperature Metrics Available | Unavailable |  | Available |
|  | 16 | External Temperature Metrics Available | Unavailable |  | Available |
|  | 17 | Other Environmental Metrics Available | Unavailable |  | Available |
| Interface – Design and Delivery | 18 | Software interface is intuitive and easy to use | Software interface is unintuitive and clunky. It is difficult to find where information is displayed or stored. Data is stored in databases/lists and is not readily visualised | 2 of the 3 of the listed requirements are met. Ie. Software is intuitive but data isn’t easily visualised. | Software interface is intuitive and flows easily between different information types/data. It is easy to find where information is displayed or stored. There are multiple ways to access information, however, there is one centralised way (through side tabs) to access anything required. Data is stored in databases that can be accessed for download, but are also readily visualised in different means. |
|  | 19 | Software is appropriate for data extraction and analysis | Software does not store location, activity and any other sent data, with timestamps and animal ID in a database OR stores but without explicit animal id or time stamp OR Stores all but download is unavailable | Software stores location, activity and any other sent data, with timestamps and animal ID in a database Download is available as a CSV file. | Software stores location, activity and any other sent data, with timestamps and animal ID in a database The database is available for download at any time. The animal id and time can be filtered with search functionality, and desired variables chosen. Download is available as a CSV file. |
|  | 20 | Interface is appropriate for “real-time” location and activity monitoring | Interface does not allow for easy access to a map for “real time” location-monitoring Interface does not publish “real time” activity (“low”, “med”, “high”) | 1 of the 2 requirements listed are met | Interface allows for easy access to a map for “real time” location-monitoring Interface does not publish “real time” activity (“low”, “med”, “high”) |
| Interface- Data Visualisation & Alerts | 21 | Animal “real time” location visualisation | Animal “real time” location data is available but not visualised |  | Animal “real time” location data is available and visualised |
|  | 22 | Animal activity data visualisation | Animal activity data is available but not visualised |  | Animal activity data is available and visualised |
|  | 23 | Environmental data (from base stations or otherwise) | Environmental data is available but not visualised | Environmental data is available, and visualised as pinpoint data | Environmental data is available, and visualised as a gradient over the landscape |
|  | 24 | Animal Reduced Activity alerts (for animal endangerment OR dropped tracker) | No alert available for changes in animal activity level | Animal activity alert triggers based on if activity level is low OR Animal activity alert triggers based on if location data is stagnant for >12 hrs | Animal activity alert triggers based on if activity level is low AND Animal activity alert triggers based on if location data is stagnant for >12 hrs |
|  | 25 | Environmentally Dangerous Conditions Alert | No alert available for dangerous environmental conditions | Dangerous environment alert triggered by extreme heat | Dangerous environment alerts are differentiated and triggered by high heat AND high heat/high winds (potential fire alert) |
| Battery | 26 | Appropriate # of fixes per day | Number of fixes per day are unreliable. Not at least 1 fix per day | Number of fixes per day is consistent At least 1 fix is made per day | Number of fixes per day is consistent More than 1 fix is available per day (0.5) At least 4 fixes are available per day (1) |
|  | 27 | Battery life is >1 year at appropriate (2) fix rate | Battery life is <2 months. | Battery life is 2-6 months. | Battery life is >6 months |
|  | 28 | Attachment style is not battery intensive and does not significantly reduce battery life | Attachment uses more than 100J | Attachment uses between 1-100J | Attachment uses less than 1J |

Animal-Specific Requirements

Table 18 Animal specific requirements for animal tracking devices and the expected thresholds a product should meet, as collated from initial requirement gathering interviews and literature reviews. These are grouped into the overarching concerns: attachment; weight; behaviour.

| Overarching Theme | # | Specific Requirement | Poor | Adequate | Excellent |
| --- | --- | --- | --- | --- | --- |
| Attachment | 1 | Attachment Style is suited to Peramelidae, and is not affected by jaw/neck/head shape, nor by behaviour | Attachment falls off or causes injury and/or death | Attachment is loose fitting or causes injury | Attachment is well-fitted, is unaffected by marsupial behaviours, and does not cause injury and/or death |
|  | 2 | Attachment in-field is non-invasive and non-stressful to animal | Requires 2 people Takes more than 15 mins post-capture Is laborious and intrusive to animal (a lot of manipulation required, tight-fitting and/or difficult to attach) | Requires 1-2 people Takes 10-15 mins post-capture Is difficult but manageable to attach, and non-intrusive | Requires 1 person Takes less than 10 mins post-capture Is simple and manageable to attach, without requiring any manipulation of the animal |
|  | 3 | Attachment is elasticated for weight fluctuations | Non-elasticated |  | Elasticated |
|  | 4 | Attachment style is delicate on skin, and does not cause rubbing resulting in skin injuries | Attachment is either too tight-fitting that blood flow is impeded, or too loose to cause movement of attachment. Material used is rough, not dermatologically tested, and is affected by moisture/heat fluctuations. Material/design is non-breathable, and traps moisture. | 2 out of 3 of the listed requirements are met. | Attachment is elasticated to adjust with weight fluctuations, where the level of elastication allows for a snug but not tight fit. Material used is soft, dermatologically tested/used in human health settings, and is unaffected by moisture/heat fluctuations. Material/design is breathable and does not trap moisture. |
|  | 5 | Attachment lasts for an extended period, to reduce intrusion to animal | Attachment last for <2 months. | Attachment last for 2-6 months. | Attachment last for >6 months |
|  | 6 | Attachment is suited to attachment of mature juveniles | Attachment does not fluctuate with weight Attachment size is non-suited to juvenile ie. length | 1 of the 2 requirements listed is met | Attachment fluctuates with weight and is a suitable size for the animal both as a juvenile and as an adult ie. for a tail attachment length of attachment is not too long for a juvenile |
|  | 7 | Attachment should not impact survivability or breeding-success. |  |  |  |
| Weight | 8 | Total System weight is appropriate for small marsupials*, especially Peramelidae | Weight is >25g | Weight is 15g to 25g | Weight is <15g |
| Behaviour | 9 | Tracker does not cause changes in behaviour | Tracker is intrusive to some degree and the animal continually attempts to remove | Tracker is intrusive to some degree, yet the animal adjusts to within 15 mins. Animal does not try to remove past this stage. | Tracker is non-intrusive when attached and the animal does not attempt to remove |
|  | 10 | Tracker is not removable by animal | Tracker is easily removed by animal (within 24 hrs) |  | Tracker cannot be easily removed by animal (within 24 hrs) |
|  | 11 | Tracker easily detaches when programmed detachment occurs | Tracker is not detached from animal within 1hr | Tracker reliably detaches from animal within 15mins-1hr | Tracker reliably detaches from animal within 15mins |

*Includes the following Families: Peramelidae, Perorycitidae, Myrmcobiidae, Dasyuridae, Potoridae, Thylacomidae, Macropodidae. 
These families include species with body mass ranging from 500-9000g.